Section 01 7839 
Project Record Documents

  - ************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 4 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) General POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable specification sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - ************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

Administrative and procedural requirements for project record documents including the following:

        - ******************************************************************************

        - The following subparagraphs are designer-choice. Include based on types of documents required for the project.

        - ******************************************************************************

Record Drawings.

Record Specifications.

Record Product Data.

Record Calculations.

Miscellaneous Record Submittals.

      - ***************************************************************************************

      - Section << 01 7700 - Closeout Procedures>> and Section << 01 7823 - Operation and Maintenance Data>>​ are not referenced in the body of this Section. Including Section << 01 7700 - Closeout Procedures>> for general closeout procedures and Section << 01 7823 - Operation and Maintenance Data>> for operation and maintenance manual requirements are a designer-choice.

      - ***************************************************************************************

RELATED REQUIREMENTS

Section << 01 3300 - Submittal Procedures>>​

Section << 01 7700 - Closeout Procedures>>

Section << 01 7823 - Operation and Maintenance Data>>

      - ***************************************************************************************

      - The following Article Reference Standards is a designer-choice.

      - ***************************************************************************************

REFERENCE STANDARDS

None

INFORMATIONAL SUBMITTALS AT CLOSEOUT

      - ***************************************************************************************

      - The following five paragraphs are designer-choice. Include based on types of documents required for the project.

      - ***************************************************************************************

Record Drawings: Comply with the following:

Submit set(s) of marked-up record Drawings to LANL Construction Subcontract Technical Representative (STR) as follows:

Initial Submittal:

            - *************************************************************

            - If project requires paper copies, include subparagraph below.

            - *************************************************************

​<< One; or  _______>>​ paper-copy set(s) of marked-up record prints.

PDF electronic files of scanned record prints and one of file prints.

Record digital data files and one set of plots.

A/E of Record shall indicate whether general scope of changes, additional information recorded, and quality of drafting are acceptable.

Final Submittal:

            - *************************************************************

            - If project requires paper copies, include subparagraph below.

            - *************************************************************

Submit ​<< one; or  _______>>​ paper-copy set of marked-up record prints.

Submit PDF electronic files of scanned record prints and one set of prints.

Submit record digital data files and one set of record digital data file plots.

Print and/or plot each drawing, whether or not changes and additional information were recorded.

Record Specifications: Submit one paper copy and annotated PDF electronic files of Project’s specifications, including addenda and subcontract modifications.

Record Product Data: Submit one paper copy and annotated PDF electronic files and directories of each submittal.

Where record Product Data are required as part of operation and maintenance manuals, submit duplicate marked-up Product Data as a component of manual.

Record Calculations: Submit one paper copy and annotated PDF electronic files of projects calculations developed or revised during construction.

Miscellaneous Record Submittals: See other Specification Sections for miscellaneous recordkeeping requirements and submittals in connection with various construction activities.  Submit one paper copy and annotated PDF electronic files and directories of each submittal.

Reports: Submit written report at scheduled construction meeting indicating items incorporated into project record documents concurrent with progress of the Work, including revisions, concealed conditions, field changes, product selections, and other notations incorporated.

PART 2 - PRODUCTS

    - **********************************************************************************************

      - The following Articles are designer-choice. Include based on types of documents required for the project.

    - **********************************************************************************************

RECORD DRAWINGS

Record Prints: Maintain one set of marked-up paper copies of the Subcontract Drawings and Shop Drawings, incorporating new and revised drawings as modifications are issued.

Preparation: Mark record prints with red colored pencil or pen to show the actual installation where installation varies from that shown originally.  Require individual or entity who obtained record data, whether individual or entity is Installer, subcontractor, or similar entity, to provide information for preparation of corresponding marked-up record prints.

Give particular attention to information on concealed elements that would be difficult to identify or measure and record later.

Accurately record information in an acceptable drawing technique.

Record data as soon as possible after obtaining it.

Record and check the markup before enclosing concealed installations.

          - **********************************************************************

          - Include following subparagraph if LANL reviewed and approved photographs authorized on Project.

          - **********************************************************************

Cross-reference record prints to corresponding archive photographic documentation.

Content: Types of items requiring marking include, but are not limited to, the following:

Dimensional changes to Drawings.

Revisions to details shown on Drawings.

Depths of foundations below first floor.

Locations and depths of underground utilities.

Revisions to routing of piping and conduits.

Revisions to electrical circuitry.

Actual equipment locations.

Duct size and routing.

Locations of concealed internal utilities.

Changes made by Change Order or Field Change Notice.

Changes made following LANL’s written orders.

Details not on the original Subcontract Drawings.

Field records for variable and concealed conditions.

Record information on the Work that is shown only schematically.

Field test records.

Inspection certificates.

Manufacturer’s certificates.

Specified installer/tradesman certificates.

Storm Water Pollution Prevention Plan.

Delegated Design Submittals

Locations of contaminated areas and/or hazardous materials (asbestos, lead, beryllium, chemicals, spills, etc.) identified during construction and are to remain in place.

          - **********************************************************************

          - The following subparagraph is a designer-choice.

          - **********************************************************************

Submittals after A/E review and approval.

Mark the Subcontract Drawings and Shop Drawings completely and accurately. Use personnel proficient at recording graphic information in production of marked-up record prints.

Mark record sets with erasable, red-colored pencil. Use other colors to distinguish between changes for different categories of the Work at same location.

Mark important additional information that was either shown schematically or omitted from original Drawings.

Note Field Change Notices numbers, alternate numbers, Change Order numbers, and similar identification, where applicable.

Record Digital Data Files: Immediately before inspection for certificate of Substantial Completion, review marked-up record prints with LANL STR and A/E of Record. When authorized, prepare a full set of corrected digital data files of the Subcontract Drawings, as follows:

Format: Same digital data software program, version, and operating system as the original Subcontract Drawings.

Format: ​​<< DWG;  DXF; or  DGN>>​​, Version ​<< ______>>​, ​<< Microsoft Windows; or  Apple MacOS>>​ operating system.

Format: Annotated PDF electronic file<< with comment function enabled; or  None - N/A>>.

Incorporate changes and additional information previously marked on record prints. Delete, redraw, and add details and notations where applicable.

Refer instances of uncertainty to A/E of Record for resolution.

A/E of Record shall furnish Construction Subcontractor one set of digital data files of the Subcontract Drawings for use in recording information.

See Section << 01 3300 - Submittal Procedures>> for requirements related to use of A/E of Record’s digital data files.

A/E of Record shall provide date file layer information.  Record markups in separate layers.

Newly Prepared Record Drawings:  Prepare new Drawings instead of preparing record Drawings where A/E of Record determines that neither the original Subcontract Drawings nor Shop Drawings are suitable to show actual installation.

New Drawings may be required when a Change Order is issued as a result of accepting an alternate, substitution, or other modification.

Consult LANL STR for proper scale and scope of detailing and notations required to record the actual physical installation and its relation to other construction. Integrate newly prepared Record Drawings into Record Drawing sets; comply with procedures for formatting, organizing, copying, binding, and submitting.

Format:  Identify and date each record Drawing; include the designation "PROJECT RECORD DRAWING" in a prominent location.

Record Prints:  Organize record prints and newly prepared record Drawings into manageable sets.  Bind each set with durable paper cover sheets. Include identification on cover sheets.

Format:  Annotated PDF electronic file << with comment function enabled; or  None - N/A>>.

Record Digital Data Files:  Organize digital data information into separate electronic files that correspond to each sheet of the Subcontract Drawings.  Name each file with the LANL project ID number and sheet identification. Include identification in each digital data file.

Identification: As follows:

Project name

Date

Designation “PROJECT RECORD DRAWINGS”

Name of A/E of Record

Name of Construction Subcontractor

RECORD SPECIFICATIONS

Preparation:  Mark Specifications to indicate the actual product installation where installation varies from that indicated in Specifications, addenda, and subcontract modifications.

Give particular attention to information on concealed products and installations that cannot be readily identified and recorded later.

Mark copy with the proprietary name and model number of products, materials, and equipment furnished, including substitutions and product options selected.

Record the name of manufacturer, supplier, installer, and other information necessary to provide a record of selections made.

For each principal product, indicate whether record Product Data has been submitted in operation and maintenance manuals instead of submitted as record Product Data.

Note related change orders​​<< , record product data,; or  None - N/A>>​​​ and record drawings where applicable.

Format: Submit record specifications as ​<< annotated PDF electronic file;  paper copy; or  scanned PDF electronic file(s) of marked-up paper copy of specifications>>​.

RECORD PRODUCT DATA

Preparation: Mark Product Data to indicate the actual product installation where installation varies substantially from that indicated in Product Data submittal.

Give particular attention to Information on concealed products and installations that cannot be readily identified and recorded later.

Include significant changes in the product delivered to project site and changes in manufacturer's written instructions for installation.

Note related Change Orders<< , record specifications,; or  None - N/A>> and record drawings where applicable.

Format: Submit record Product Data as << annotated PDF electronic file;  paper copy; or  scanned PDF electronic file(s) of marked-up paper copy of Product Data>>.

Include record Product Data directory organized by Specification Section number and title, electronically linked to each item of record Product Data.

RECORD CALCULATIONS

Assemble calculation records in connection with actual performance of the Work. Bind or file calculation records and identify each, ready for continued use and reference.

Format: Submit calculation record submittals as << annotated PDF electronic file;  paper copy; or  scanned PDF electronic file(s) of marked-up paper copy of calculations>>.

Include calculation record submittals directory organized by Specification Section number and title, electronically linked to each item of calculation record submittal.

MISCELLANEOUS RECORD SUBMITTALS

Assemble miscellaneous records required by other Specification Sections for miscellaneous record keeping and submittal in connection with actual performance of the Work.  Bind or file miscellaneous records and identify each, ready for continued use and reference.

Format: Submit miscellaneous record submittals as ​​<< annotated PDF electronic file;  paper copy; or  scanned PDF electronic file(s) of marked-up paper copy of miscellaneous record submittals>>​​.

PART 3 - EXECUTION

RECORDING AND MAINTENANCE

Recording:  Maintain one copy of each submittal during the construction period for project record document purposes.  Post changes and revisions to project record documents as they occur; do not wait until end of Project.

Maintenance of Record Documents and Samples: Store record documents and Samples in the field office apart from the Subcontract Documents used for construction. Do not use project record documents for construction purposes. Maintain record documents in good order and in a clean, dry, legible condition, protected from deterioration and loss. Provide access to project record documents for LANL STR and A/E of Record’s reference during normal working hours.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 01 7839, REV. 4, DATED APRIL 15, 2026.