Section 08 1213 
Hollow Metal Frames

  - *************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 5 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects. For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 specifications and quality sections.

  - *************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain paragraphs for type(s) of frame(s) and accessories selected for project based on project-scope. Selections here update the entire Section.

      - ***************************************************************************************

Interior Standard Steel Frames - ​​​<< Click to select options from the drop down menu;  Fire-rated Smoke-control and draft-control assemblies;  Fire-rated Oversize fire-rated door assemblies;  Fire-rated Borrowed-lite assemblies; and  Non-fire rated doors>>​​​

Exterior Standard Steel Frames - ​​​<< Click to select options from the drop down menu;  Fire-rated Smoke-control and draft-control assemblies;  Fire-rated Oversize fire-rated door assemblies;  Fire-rated Borrowed-lite assemblies; and  Non-fire rated doors>>​​​

Accessories - ​​​​<< Click to select options from the drop down menu;  Borrowed Lites;  Hollow Metal Panels;  Door Hardware; and  Frame Anchors>>​​​​

RELATED REQUIREMENTS

Section << 08 7100 - Door Hardware>>

Section ​<< 08 8000 - Glazing>>​

Section << 09 9100 - Painting>>

REFERENCE STANDARDS

ANSI/SDI A250.3 - Test Procedure and Acceptance Criteria for Factory Applied Finish Coatings for Steel Doors and Frames.

ANSI/SDI A250.4 - Test Procedure and Acceptance Criteria for Physical Endurance for Steel Doors, Frames and Frame Anchors.

ANSI/SDI A250.6 - Recommended Practice for Hardware Reinforcing on Standard Steel Doors and Frames.

ANSI/SDI A250.8 - Specifications for Standard Steel Doors and Frames (SDI-100).

ANSI/SDI A250.10 - Test Procedure and Acceptance Criteria for Prime Painted Steel Surfaces for Steel Doors and Frames.

ANSI/SDI A250.11 - Recommended Erection Instructions for Steel Frames.

ASTM A153/A153M - Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware.

ASTM A1008/A1008M - Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Required Hardness, Solution Hardened, and Bake Hardenable.

ASTM A1011/A1011M - Standard Specification for Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength.

ASTM C665 - Standard Specification for Mineral-Fiber Blanket Thermal Insulation for Light Frame Construction and Manufactured Housing.

ASTM E136 - Standard Test Method for Assessing Combustibility of Materials Using a Vertical Tube Furnace at 750 Degrees C.

BHMA A156.115 - Hardware Preparation in Steel Doors and Frames.

NFPA 80 - Standard for Fire Doors and Other Opening Protectives.

NFPA 105 - Standard for Smoke Door Assemblies and Other Opening Protectives.

NFPA 252 - Standard Methods of Fire Tests of Door Assemblies.

NFPA 257 - Standard on Fire Test for Window and Glass Block Assemblies.

UL 9 - Standard for Fire Tests of Window Assemblies.

UL 10C - Standard for Positive Pressure Fire Tests of Door Assemblies.

UL 1784 - Standard for Air Leakage Tests of Door Assemblies and Other Opening Protectives.

DEFINITIONS

For definitions and acronyms, refer to COE Glossary of Terms.

Minimum Thickness: Minimum thickness of base metal without coatings according to <<ANSI/SDI A250.8>>.

COORDINATION

Coordinate anchorage installation for hollow metal frames. Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors. Deliver such items to Project site in time for installation.

      - ***************************************************************************************

      - Retain the following paragraph for door hardware.

      - ***************************************************************************************

Coordinate requirements with the LANL Subcontract Technical Representative (STR) for installation of door hardware, electrified door hardware, and access control and security systems.

      - ***************************************************************************************

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

ACTION SUBMITTALS

Product Data: For each type of product.

Include construction details, material descriptions, fire-resistance ratings, and finishes.

Shop Drawings: Include the following:

Elevations of each frame type.

Frame details for each frame type, including dimensioned profiles and metal thicknesses.

Locations of reinforcement and preparations for hardware.

Details of each different wall opening condition.

Details of electrical raceway and preparation for electrified hardware, access control systems, and security systems.

Details of anchorages, joints, field splices, and connections.

Details of accessories.

Details of moldings and removable stops.

      - ***************************************************************************************

      - Retain the following two paragraphs when project requires samples.

      - ***************************************************************************************

Samples for Initial Selection: For hollow metal frames with factory-applied color finishes.

Samples for Verification

Finishes: For each type of exposed finish required, prepared on Samples of not less than 3 by 5 inches.

Fabrication: Prepare Samples approximately 8 by 10 inches to demonstrate compliance with requirements for quality of materials and construction

Show profile, corner joint, floor and wall anchors, and silencers. Include separate section showing fixed hollow-metal panels and glazing, if applicable.

Product Schedule: For hollow metal frames, prepared by or under the supervision of supplier, using same reference numbers for details and openings as those on Drawings. Coordinate with final door hardware schedule.

INFORMATIONAL SUBMITTALS

Product Test Reports: For each type of hollow metal frame assembly, for tests performed by a qualified testing agency.

      - ***************************************************************************************

      - Retain the following paragraph when project requires oversize construction certificate for fire-rated door assemblies.

      - ***************************************************************************************

Oversize Construction Certification: For assemblies required to be fire-rated and exceeding limitations of labeled assemblies.

QUALITY ASSURANCE

Provide frames complying with <<ANSI/SDI A250.8>> and as herein specified.

      - ***************************************************************************************

      - Retain the following paragraph when fire-rated door assemblies are in project-scope.

      - ***************************************************************************************

Fire-Rated Door Assemblies: Where fire-rated door assemblies are indicated or required, provide fire-rated frame assemblies that comply with <<NFPA 80>>.

DELIVERY, STORAGE, AND HANDLING

Deliver hollow metal work cartoned or crated to provide protection from damage during transit and job storage. Provide additional protection to prevent damage to factory-finished units.

Inspect hollow metal work upon delivery for damage. Minor damages may be repaired provided finish items are equal in all respects to new work and acceptable to the LANL STR; otherwise, remove and replace.

Store frames at building site under cover. Store vertically, place units on minimum 4 inches high wood blocking. Do not use non-vented plastic or canvas shelters which could create a humidity chamber.

PART 2 - PRODUCTS

    - **********************************************************************************************

      - Retain the following Article Performance Requirements when fire-rated assemblies and/or fire-rated borrowed lite assemblies are in project-scope. The following paragraphs in the Article are designer-choice. Retain the first paragraph when fire-rated assemblies are required for project. Retain the second paragraph when fire-rated borrowed-lite assemblies are required for project.

    - **********************************************************************************************

PERFORMANCE REQUIREMENTS

Fire-Rated Assemblies: Complying with <<NFPA 80>> and listed and labeled by a qualified testing agency acceptable to LANL for fire-protection ratings and temperature-rise limits, based on testing at positive pressure according to <<NFPA 252>> or <<UL 10C>>.

        - ******************************************************************************

        - Retain the following subparagraphs based on type of fire-rated assemblies in project-scope.

        - ******************************************************************************

Smoke-Control and Draft-Control Assemblies: Provide assemblies with gaskets listed and labeled for smoke and draft control by a qualified testing agency acceptable to LANL, based on testing according to <<UL 1784>> and installed in compliance with <<NFPA 105>>.

Oversize Fire-Rated Door Assemblies: For units exceeding sizes of tested assemblies, provide certification by a qualified testing agency that doors comply with standard construction requirements for tested and labeled fire-rated door assemblies except for size.

Fire-Rated, Borrowed-Lite Assemblies: Complying with <<NFPA 80>> and listed and labeled by a qualified testing agency acceptable to LANL, for fire-protection ratings indicated, based on testing according to <<NFPA 257>> or <<UL 9>>.

MATERIALS

      - ***************************************************************************************

      - Retain the following two paragraphs for frames/accessories material based on project-specific requirements.

      - ***************************************************************************************

Cold-Rolled Steel Sheet: <<ASTM A1008/A1008M>>​, Commercial Steel (CS), Type B; suitable for exposed applications.

Hot-Rolled Steel Sheet: <<ASTM A1011/A1011M>>, Commercial Steel (CS), Type B; free of scale, pitting, or surface defects; pickled and oiled.

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain the first two paragraphs for fasteners selected based on project-specific requirements. Retain the third paragraph for insulation in door frames when required for project. Retain the fourth paragraph for glazing when required for project.

      - ***************************************************************************************

Inserts, Bolts, and Fasteners: Hot-dip galvanized according to <<ASTM A153/A153M>> or stainless steel.

Power-Actuated Fasteners in Concrete: Fastener system of type suitable for application indicated, fabricated from corrosion-resistant materials, with clips or other accessory devices for attaching hollow metal frames of type indicated.

Mineral-Fiber Insulation: <<ASTM C665>>, Type I (blankets without membrane facing); consisting of fibers manufactured from slag or rock wool; with maximum flame-spread and smoke-developed indexes of 25 and 50, respectively; passing <<ASTM E136>> for combustion characteristics.

Glazing: Comply with requirements in Section << 08 8000 - Glazing>>.

      - ***************************************************************************************

      - Retain the following Article when interior standard steel frames are in project-scope.

      - ***************************************************************************************

INTERIOR STANDARD STEEL FRAMES

Heavy-Duty Frames: <<ANSI/SDI A250.8>>, Level 2; <<ANSI/SDI A250.4>>, Level B.

Materials: Steel sheet, minimum thickness of 0.053 inch.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain when required for project.

        - ******************************************************************************

​​​​<< Sidelite;  Transom; or  Sidelite and Transom>>​​​​ Frames: Fabricated from same thickness material as adjacent door frame.

Construction: Full profile welded.

Exposed Finish: << Prime Coat; or  Factory Finish>>.

      - ***************************************************************************************

      - Retain the following Article when exterior standard steel frames are in project-scope.

      - ***************************************************************************************

EXTERIOR STANDARD STEEL FRAMES

Extra-Heavy-Duty Frames: <<ANSI/SDI A250.8>>, Level 3; <<ANSI/SDI A250.4>>, Level A.

Materials: Steel sheet, minimum thickness of 0.053 inch.

Construction: Full profile welded.

Exposed Finish: << Prime Coat; or  Factory Finish>>.

      - ***************************************************************************************

      - Retain the following Article when borrowed lites are selected for project.

      - ***************************************************************************************

BORROWED LITES

Frame: Steel sheet, minimum thickness 0.053 inch.

Construction: Full profile welded.

Fabricate in one piece except where handling and shipping limitations require multiple sections. Where frames are fabricated in sections due to shipping or handling limitations, provide alignment plates or angles at each joint, fabricated of metal of same or greater thickness as frames.

Provide countersunk, flat-head or oval-head exposed screws and bolts for exposed fasteners unless otherwise indicated.

      - ***************************************************************************************

      - Retain the following Article when hollow metal panels are selected for project.

      - ***************************************************************************************

HOLLOW METAL PANELS

Provide hollow metal panels of same materials, construction, and finish as adjacent door assemblies.

      - ***************************************************************************************

      - Retain the following Article for door hardware.

      - ***************************************************************************************

DOOR HARDWARE

Refer to Section << 08 7100 - Door Hardware>> for door hardware for hollow metal doors.

      - ***************************************************************************************

      - Retain the following Article for frame anchors required for installation of door frames.

      - ***************************************************************************************

FRAME ANCHORS

Jamb Anchors:

Post-installed Expansion Anchor: Select anchor type required by applicable door and frame standard, and suitable for performance level indicated. Minimum 3/8 inch diameter bolts with expansion shields or inserts, with manufacturer's standard pipe spacer.

Quantity: Minimum of three anchors per jamb, with one additional anchor for frames with no floor anchor. Provide one additional anchor for each 24 inches, or portion thereof, of frame height above 7 feet.

Floor Anchors:

Provide floor anchors for each jamb and mullion that extends to floor.

        - ******************************************************************************

        - Retain the following subparagraph if a concrete floor with underlayment is present.

        - ******************************************************************************

Floor Anchors for Concrete Slabs with Underlayment: Adjustable-type anchors with extension clips, allowing not less than 2-inch height adjustment. Terminate bottom of frames at top of underlayment.

      - ***************************************************************************************

      - Retain the following paragraph when exterior door frames are in project-scope.

      - ***************************************************************************************

For anchors built into exterior walls, use stainless steel.

FABRICATION

Construct hollow metal frames to comply with standards indicated for materials, fabrication, hardware locations, hardware reinforcement, tolerances, and clearances, and as specified.

Hollow Metal Frames: Fabricate in one piece except where handling and shipping limitations require multiple sections. Where frames are fabricated in sections, provide alignment plates or angles at each joint, fabricated of metal of same or greater thickness as frames.

        - ******************************************************************************

        - The following subparagraph is a designer-choice. Retain when required for project.

        - ******************************************************************************

<< Sidelite;  Transom; or  Sidelite and Transom>> Frames: Provide closed tubular members with no visible face seams or joints, fabricated from same material as door frame. Fasten members at crossings and to jambs by welding<< , or by rigid mechanical anchors; or  None - N/A>>.

Provide countersunk, flat-head or oval-head exposed screws and bolts for exposed fasteners unless otherwise indicated.

Door Silencers: Except on weather-stripped frames, drill stops to receive door silencers as follows. Keep holes clear during construction.

Single-Door Frames: Drill stop in strike jamb to receive three door silencers.

Double-Door Frames: Drill stop in head jamb to receive two door silencers.

      - ***************************************************************************************

      - Retain the following paragraph for door hardware.

      - ***************************************************************************************

Hardware Preparation: Factory prepare hollow metal frames to receive templated mortised hardware, and electrical wiring; include cutouts, reinforcement, mortising, drilling, and tapping according to <<ANSI/SDI A250.6>>, the Door Hardware Schedule, and templates.

Reinforce frames to receive non-templated, mortised, and surface-mounted hardware.

Comply with <<BHMA A156.115>> for preparing hollow metal frames for hardware.

      - ***************************************************************************************

      - Retain the following paragraph when glazing required for project.

      - ***************************************************************************************

Glazed Lites: Provide stops and moldings around glazed lites where indicated. Form corners of stops and moldings with mitered hairline joints.

Provide square stops unless otherwise indicated.

Multiple Glazed Lites: Provide fixed and removable stops and moldings so that each glazed lite is capable of being removed independently.

Provide fixed frame moldings on outside of exterior and on secure side of interior frames. Provide loose stops and moldings on inside of hollow metal frames.

Coordinate rabbet width between fixed and removable stops with glazing and installation types indicated.

Provide stops for installation with countersunk flat-head or oval-head machine screws spaced uniformly not more than 9 inches o.c. and not more than 2 inches o.c. from each corner.

CLEARANCES

Unless noted otherwise, clearances are subject to tolerance of plus or minus 1/32 inch.

Clearance between Door and Frame Head and Jambs: 1/8 inch.

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Retain the first paragraph when fire-rated doors are in project-scope. Retain the second paragraph when non-fire rated doors are in project-scope.

      - ***************************************************************************************

Clearance between Meeting Edges of Pairs of Fire-Rated Doors: 1/8 inch plus or minus 1/16 inch.

Clearance between Meeting Edges of Non-Fire Rated Pairs of Doors: 1/8 inch to 1/4 inch.

Clearance at Bottom: 5/8 inch.

Clearance between Door Face and Door Stop: 1/16 inch to 1/8 inch.

STEEL FINISHES

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Retain the first paragraph when prime coat finish selected for project. Retain the second paragraph when factory finish selected for project.

      - ***************************************************************************************

Prime Finish: Clean, pretreat, and apply manufacturer's standard primer.

Shop Primer: Manufacturer's standard, fast-curing, lead-free and chromate-free primer complying with <<ANSI/SDI A250.10>>; recommended by primer manufacturer for substrate; compatible with substrate and field-applied coatings despite prolonged exposure.

Factory Finish: Clean, pretreat, and apply manufacturer's standard two-coat, baked-on finish consisting of prime coat and thermosetting topcoat, complying with <<ANSI/SDI A250.3>>.

Color and Gloss: As selected by Architect from manufacturer's full range.

PART 3 - EXECUTION

PREPARATION

Remove welded-in shipping spreaders installed at factory. Restore exposed finish by grinding, filling, and dressing, as required to make repaired area smooth, flush, and invisible on exposed faces. Touch up factory-applied finishes where spreaders are removed.

      - ***************************************************************************************

      - Retain the following paragraph for door hardware.

      - ***************************************************************************************

Drill and tap frames to receive non-templated, mortised, and surface-mounted door hardware.

INSTALLATION

General: Install hollow metal frames plumb, rigid, properly aligned, and securely fastened in place. Comply with approved Shop Drawings and with manufacturer's written instructions.

Hollow Metal Frames: Comply with <<ANSI/SDI A250.11>>.

Set frames accurately in position; plumbed, aligned, and braced securely until permanent anchors are set. After wall construction is complete, remove temporary braces without damage to completed Work.

Where frames are fabricated in sections, field splice at approved locations by welding face joint continuously; grind, fill, dress, and make splice smooth, flush, and invisible on exposed faces. Touch-up finishes.

Install frames with removable stops located on secure side of opening.

        - ******************************************************************************

        - Retain the following subparagraph when fire-rated borrowed lite assemblies are in project-scope.

        - ******************************************************************************

Fire-Rated Openings: Install frames according to <<NFPA 80>>.

Floor Anchors: Secure with post-installed expansion anchors.

Floor anchors may be set with power-actuated fasteners instead of post-installed expansion anchors if indicated and approved on Shop Drawings.

        - ******************************************************************************

        - Retain the following subparagraph when mineral insulation is required inside door frames.

        - ******************************************************************************

Solidly pack mineral-fiber insulation inside frames.

        - ******************************************************************************

        - The following two subparagraphs are designer-choice. Retain the first subparagraph if masonry walls are present. Retain the second subparagraph if concrete or masonry construction is present.

        - ******************************************************************************

Masonry Walls: Coordinate installation of frames to allow for solidly filling space between frames and masonry with grout or mortar.

In-Place Concrete or Masonry Construction: Secure frames in place with post-installed expansion anchors.​<<  Countersunk anchors, and fill and make smooth, flush, and invisible on exposed faces.; or  None - N/A>>​

Installation Tolerances: Adjust hollow metal frames to the following tolerances:

Squareness: Plus or minus 1/16 inch, measured at door rabbet on a line 90 degrees from jamb perpendicular to frame head.

Alignment: Plus or minus 1/16 inch, measured at jambs on a horizontal line parallel to plane of wall.

Twist: Plus or minus 1/16 inch, measured at opposite face corners of jambs on parallel lines, and perpendicular to plane of wall.

Plumbness: Plus or minus 1/16 inch, measured at jambs at floor.

      - ***************************************************************************************

      - Retain the following paragraph when glazing required for project.

      - ***************************************************************************************

Glazing: Comply with installation requirements in Section ​<< 08 8000 - Glazing>>​ and with hollow metal manufacturer's written instructions.

CLEANING AND TOUCHUP

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Retain the first paragraph when prime coat finish selected for project. Retain the second paragraph when factory finish selected for project.

      - ***************************************************************************************

Prime Coat Touchup: Immediately after erection, sand smooth rusted or damaged areas of prime coat and apply touchup of compatible air-drying, rust-inhibitive primer.

Factory Finish Touchup: Clean abraded areas and repair with same material used for factory finish according to manufacturer's written instructions.

Touchup Painting: Cleaning and touchup painting of abraded areas of paint are specified in Section ​<< 09 9100 - Painting>>​.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 08 1213, REV. 5, DATED APRIL 15, 2026.