Section 10 2800 
Toilet and Bath Accessories

  - ************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 4 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Architectural POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable Sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects. For ML1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 specifications and quality sections.

  - ************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

Toilet and Bath Accessories:

        - ******************************************************************************

        - The following subparagraphs are designer-choice. Retain subparagraphs for toilet and bath accessories based on project-scope.

        - ******************************************************************************

Washroom accessories - ​​​​​​<< Click to select accessories from drop down menu;  Liquid Soap Dispenser;  Paper Towel (Roll) Dispenser;  Toilet Tissue (Roll) Dispenser;  Coat Hook;  Fold-Down Purse Shelf;  Grab Bar;  Mirror Unit;  Sanitary Napkin/Tampon Dispenser;  Sanitary Napkin Disposal Unit;  Seat Cover Dispenser; and  Waste Receptacle>>​​​​​​

Shower room accessories.

Warm air dryers.

Under lavatory guards.

Custodial accessories - ​​​​<< Click to select accessories from drop down menu;  Utility Shelf; and  Mop and Broom Holder>>​​​​ .

RELATED REQUIREMENTS

Section << 06 1000 - Rough Carpentry>>

REFERENCE STANDARDS

ABA Standards - ABA Accessibility Standards.

ASTM A153/A153M - Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware.

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

ASTM A666/A666M - Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate, and Flat Bar.

ASTM A1008/A1008M - Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Required Hardness, Solution Hardened, and Bake Hardenable.

ASTM B16/B16M - Standard Specification for Free-Cutting Brass Rod, Bar and Shapes for Use in Screw Machines.

ASTM B30 - Standard Specification for Copper Alloys in Ingot and Other Remelt Forms.

ASTM B36/B36M - Standard Specification for Brass Plate, Sheet, Strip, and Rolled Bar.

ASTM B456 - Standard Specification for Electrodeposited Coatings of Copper Plus Nickel Plus Chromium and Nickel Plus Chromium.

ASTM C1503 - Standard Specification for Silvered Flat Glass Mirror.

ASTM F446 - Standard Consumer Safety Specification for Grab Bars and Accessories Installed in the Bathing Area.

NFPA 70 - National Electrical Code.

NSF P335 - Hygienic Commercial Hand Dryers.

UL (DIR) - Online Certifications Directory.

UL 507 - Electric Fans.

COORDINATION

Coordinate accessory locations with other work to prevent interference with clearances required for access by people with disabilities, and for proper installation, adjustment, operation, cleaning, and servicing of accessories.

Deliver inserts and anchoring devices set into concrete or masonry as required to prevent delaying the work.

Coordinate the work with placement of << internal wall reinforcement; and  reinforcement of toilet partitions>> to receive anchor attachments.

      - ***************************************************************************************

      - Edit submittals to include only those absolutely necessary to assure requirements and features that are important for the specific project will be met.

      - ***************************************************************************************

ACTION SUBMITTALS

Product Data: For each type of product indicated include the following:

Construction details and dimensions.

Anchoring and mounting requirements, including requirements for cutouts in other work and substrate preparation.

Material and finish descriptions.

Features that will be included for Project.

Manufacturer’s warranty.

      - ***************************************************************************************

      - Retain the following paragraph if full size samples of accessories is required for project.

      - ***************************************************************************************

Samples: Full size, for each accessory item to verify design, operation, and finish requirements.

Approved full-size Samples will be returned and may be used in the Work.

Product Schedule: Indicating types, quantities, sizes, and installation locations by room of each accessory required.

Identify locations using room designations indicated.

Identify products using designations indicated.

INFORMATIONAL SUBMITTALS

      - ***************************************************************************************

      - Retain the following paragraph if a sample of special warranty is required  when mirrors are in project-scope.

      - ***************************************************************************************

Warranty: Sample of special warranty.

At Closeout: Maintenance Data for toilet and bath accessories to include in maintenance manuals.

QUALITY ASSURANCE

Source Limitations: For products listed together in the same PART 2 articles, obtain products from single source or single manufacturer.

Electrical Components, Devices, and Accessories: Listed and labeled as defined in <<NFPA 70>>​, by a qualified testing agency, and marked for intended location and application.

      - ***************************************************************************************

      - Retain the following paragraph when dryers are included in project-scope.

      - ***************************************************************************************

Warm Air Dryers

Manufacturer Qualifications: Company specializing in manufacturing electric hand dryers with 20 years minimum experience.

Product Certification: ETL listed in accordance with <<UL 507>>​ and <<NSF P335>> compliant.

Equipment certified by <<UL (DIR)>>, with UL labels.

DELIVERY, STORAGE, AND HANDLING

Store products in manufacturer’s unopened packaging until ready for installation.

Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

FIELD CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer’s absolute limits.

      - ***************************************************************************************

      - Retain the following Article when mirrors are in project-scope.

      - ***************************************************************************************

WARRANTY

Special Mirror Warranty: Manufacturer’s standard form in which manufacturer agrees to replace mirrors that develop visible silver spoilage defects and that fail in materials or workmanship within specified warranty period.

Warranty period: << 15; or  _______>> years from date of substantial completion.

PART 2 - PRODUCTS

    - **********************************************************************************************

      - Retain the following Article for grab bars and shower room accessories. Edit this article to include other system design requirements, as required. Edit prescriptive descriptions elsewhere in this Section accordingly.

      - This article includes loads for ICC code compliance. Edit required loads when other codes govern the project.

      - Selection of "dressing room bench seats" option in paragraph below updates the rest of the Section.

    - **********************************************************************************************

DESIGN REQUIREMENTS

Design grab bars, << shower seats; and  dressing room bench seats>>, and attachments to resist minimum 250 lbf concentrated load applied at any point in any direction.

MATERIALS

      - ***************************************************************************************

      - The following five paragraphs are designer-choices. Select the material(s) for toilet and bath accessories based on project-specific requirements.

      - ***************************************************************************************

Stainless Steel: <<ASTM A666/A666M>>, Type 304, 0.031-inch minimum nominal thickness unless otherwise indicated.

Brass: <<ASTM B36/B36M>>, flat products; <<ASTM B16/B16M>>, rods, shapes, forgings, and flat products with finished edges; or <<ASTM B30>>, castings.

Steel Sheet: <<ASTM A1008/A1008M>>, Designation CS (cold rolled, commercial steel), 0.036-inch minimum nominal thickness.

        - ******************************************************************************

        - Retain the following subparagraph when required for project.

        - ******************************************************************************

Chrome Plating: <<ASTM B456>>​, Service Condition Number SC2 (moderate service).

Galvanized-Steel Sheet: <<ASTM A653/A653M>>, with G60 hot-dip zinc coating.

Galvanized-Steel Mounting Devices: <<ASTM A153/A153M>>, hot-dip galvanized after fabrication.

Fasteners: Screws, bolts, and other devices of same material as accessory unit and tamper-and-theft resistant where exposed, and of galvanized steel where concealed.

      - ***************************************************************************************

      - Retain the following paragraph when mirrors are in project-scope.

      - ***************************************************************************************

Mirrors: <<ASTM C1503>>, Mirror Glazing Quality, clear-glass mirrors, nominal 6.0 mm thick.

      - ***************************************************************************************

      - Retain the following paragraph when sanitary napkin/tampon dispensers and/or seat cover dispensers are in project-scope.

      - ***************************************************************************************

ABS Plastic: Acrylonitrile-butadiene-styrene resin formulation.

      - ***************************************************************************************

      - The following five Articles Washroom Accessories, Shower Room Accessories, High Efficiency Warm Air Dryers, Under Lavatory Guards and Custodial Accessories are designer-choice. Retain Articles for toilet and bath accessories selected for project.

      - ***************************************************************************************

WASHROOM ACCESSORIES

Manufacturers:

A&J Washroom Accessories, Inc.

American Specialties, Inc.

Bobrick Washroom Equipment, Inc.

Bradley Corporation.

GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

Tubular Specialties Manufacturing, Inc.

​<< Insert manufacturer’s name>>​.

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain paragraphs corresponding to washroom accessories selected for project.

      - The following toilet accessories (liquid soap dispenser, paper towel dispenser, toilet paper dispenser) are standardized supplies and accessories provided by the LANL janitorial service (Government Furnished Equipment (GFE)).

      - Design Agency in conjunction with the LANL Project Engineer to determine if following toilet accessories are to be provided by the janitorial service or Construction Subcontractor (installed by the Construction Subcontractor in either case).

      - ***************************************************************************************

Liquid Soap Dispenser: << GFE; or  None - N/A>>

Manufacturer and Product: Unisource TM Manual Wall Mount Foam System. The system includes a Unisource™ Foaming Soap Wall Mount Dispenser (U23095) and Respect™ 1250mL Foaming Hand Soap Refill (U24695). No substitutions.

Paper Towel (Roll) Dispenser: ​<< GFE; or  None - N/A>>​

Manufacturer and Product: Kimberly-Clark Professional IN-SIGHT® SANITOUCH® Hard Roll Towel Dispenser (09990). No substitutions.

Toilet Tissue (Roll) Dispenser: ​<< GFE; or  None - N/A>>​

Manufacturer and Product: Kimberly-Clark Professional IN-SIGHT* Cored JRT Combo Unit Bath Tissue Dispenser (09551). No substitutions.

Coat Hook:

Manufacturer and Product: ​<< Insert manufacturer’s name; product name or designation>>​.

Description:​<< Double; or  Single>>​-prong unit.

Material and Finish: ​​​​​​​<< Solid brass, polished;  Polished brass-plated zinc alloy (zamac);  Polished chrome-plated brass;  Polished chrome-plated zinc alloy (zamac);  Stainless steel, No. 4 finish (satin);  Stainless steel, No. 7 finish (polished); or  _______>>​​​​​​​​.

Fold-Down Purse Shelf:

Manufacturer and Product: ​<< Insert manufacturer’s name; product name or designation>>​.

Description: Hinged unit with spring-loaded shelf that automatically returns to vertical position.

Nominal size: ​​​<< 15 inches long by 5-1/2 inches wide; or  _______>>​​​. ​​

Material and Finish: ​​<< Chrome-plated, cast-zinc alloy (zamac) with stippled finish on tray or stainless steel, No. 4 finish (satin);  Chrome-plated, cast-zinc alloy (zamac) with stippled finish on tray and bright chrome finish on edges; or  Stainless steel, No. 4 finish (satin)>>​​.

Grab Bar:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Mounting: Flanges with << concealed; or  exposed>> fasteners.

Material and finish: Stainless steel, 0.05 inch thick; Smooth, No. 4 finish (satin)<< on end and slip-resistant texture in grip area; or  None - N/A>>

Outside Diameter: ​<< 1-1/4 inches; or  1-1/2 inches>>​.

Configuration and Length: ​​​<< As indicated on Drawings;  Straight, 36 inches long; or  ______>>​​​. ​​

Mirror Unit:

Manufacturer and Product:  ​<< Insert manufacturer’s name; product name or designation>>​.

Frame: ​​<< Stainless steel angle, 0.05 inch thick;  Stainless steel channel;  Stainless steel, fixed tilt; or  Stainless steel, adjustable tilt>>​​.

Corners: ​<< Manufacturer’s standard;  Mitered and mechanically interlocked; or  Welded and ground smooth>>​.

Integral Shelf: 5 inches deep.

Hangers: Product rigid, tamper-resistant and theft-resistant installation, using method indicated below:

          - **********************************************************************

          - Select one of the following installation methods below.

          - **********************************************************************

One-piece, galvanized-steel, wall-hanger device with spring-action locking mechanism to hold mirror unit in position with no exposed screws or bolts.

          - *** OR ***

Wall bracket of galvanized steel, equipped with concealed locking devices requiring a special tool to remove.

Size: ​​​​<< As indicated on Drawings; or  _______>>​​​​.

Material and Finish:

Dispenser Face: ​​​​<< Stainless steel, No. 4 finish (satin);  Stainless steel, No. 7 finish (polished); or  _______>>​​​​. ​​

Cabinet: ​​​​<< Steel with corrosion-resistant finish; or  _______>>​​​​. ​​

Sanitary Napkin/Tampon Dispenser:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Type: ​<< Sanitary napkin; or  Sanitary napkin and tampon>>​.

Mounting: ​​<< Fully recessed, designed for 4 inch wall depth;  Semi-recessed; or  Surface Mounted>>​​.

Capacity: ​<< Insert description>>​.

Operation: No coin (free).

Material and Finish: << Stainless steel, No. 4 finish (satin); or  _______>>.

Lockset: Tumbler type with separate lock and key for coin box.

Sanitary Napkin Disposal Unit:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Mounting: << Recessed;  Partition mounted, dual access; or  Surface mounted>>.

Door or Cover: Self-closing, disposal-opening cover << and hinged face panel with tumbler lockset; or  None - N/A>>.

Receptacle: Removable.

Material and Finish: ​​<< Stainless steel, No. 4 finish (satin);  ABS plastic, gray; or  _______>>​​. ​​

Seat Cover Dispenser:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Mounting: << Surface mounted;  Recessed; or  Partition mounted, dual access>>.

Minimum Capacity: ​​<< 250;  500; or  _______>>​​ seat covers.

Material and Finish: << Stainless steel, No. 4 finish (satin);  ABS plastic, gray; or  _______>>.

Lockset: Tumbler type.

Waste Receptacle:

Manufacturer and Product: ​<< Insert manufacturer’s name: product name or designation>>​.

Mounting: ​​​<< Open top, recessed;  Self-closing disposal-opening cover, recessed;  Semi-recessed;  Surface mounted;  Wall mounted for corner installation;  Freestanding;  Undercounter; or  _______>>​​​. ​​

Minimum Capacity: ​​<< Insert gal. (L)>>​​.

Material and Finish: ​<< Stainless steel, No. 4 finish (satin); or  ______>>​.

Liner: ​​​<< Reusable vinyl liner; or  ______>>​​​. ​​

Lockset: Tumbler type for waste-receptacle.

SHOWER ROOM ACCESSORIES

Manufacturers:

A&J Washroom Accessories, Inc.

American Specialties, Inc.

Bobrick Washroom Equipment, Inc.

Bradley Corporation.

GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

Tubular Specialties Manufacturing, Inc.

​​<< Insert manufacturer’s name>>​​.

      - ***************************************************************************************

      - Retain the following paragraph when dressing room bench seats are in project-scope.

      - ***************************************************************************************

Dressing Room Bench Seat:

Manufacturer and Product: ​​<< Insert manufacturer’s name; product name or designation>>​​.

Configuration: Rectangular seat.

Seat: ​​​​<< Stainless steel, No. 4 finish (satin) 0.05-inch minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth; or  _______>>​​​​. ​​

Material and Finish: ​<< Stainless steel, No. 4 finish (satin); or  _______>>​.

Dimensions: ​​​<< Insert dimensions>>​​​.

Folding Shower Seat:

Manufacturer and Product: ​<< Insert manufacturer’s name; product name or designation>>​.

Configuration: ​​​<< L-shaped seat, designed for wheelchair access;  Rectangular seat;  Triangular, corner-type seat; or  Stainless-steel seat designed to fold into recessed-mounted, stainless-steel wall box>>​​​.

Seat: ​​​<< Phenolic or polymeric composite of slat-type or one-piece construction in color as selected by Architect;  White vinyl padded seat;  Stainless steel, No. 4 finish (satin) 0.05-inch minimum nominal thickness; with single-piece, pan-type construction and edge seams welded and ground smooth; or  _______>>​​​. ​​

Material and Finish: << Stainless steel, No. 4 finish (satin); or  _______>>.

Dimensions: ​​<< Insert dimensions>>​​.

Shower Curtain Rod:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Description: ​<< 1 inch OD; fabricated from nominal 0.0375 inch thick Stainless steel; or  1-1/4 inch OD; fabricated from nominal 0.05 inch thick Stainless steel>>​.

Mounting Flanges: ​​<< Stainless-steel flanges designed for exposed fasteners; or  _______>>​​. ​​

Finish: ​​<< No. 4 (satin); or  _______>>​​. ​​

Shower Curtain: ​​​​​

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Size: Minimum << 6 inches; or  12 inches>> wider than opening by 72 inches high.

Material: ​​<< Vinyl, minimum 0.006 inch thick, opaque, matte;  Duck, minimum 8 oz, white, 100 percent cotton;  Nylon-reinforced vinyl, minimum 10 oz. or 0.008-inch-thick vinyl, with integral antibacterial agent; or  _______>>​​.

Color: ​​<< White;  Green;  As selected from Manufacturer’s full range; or  _______>>​​. ​​

Grommets: Corrosion resistant at minimum 6 inches o.c. through top hem.

Shower Curtain Hooks: Chrome-plated or stainless steel, spring wire curtain hooks with snap fasteners, sized to accommodate specified curtain rod. Provide one hook per curtain grommet.

Soap Dish:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Description: ​<< with; or  without>>​ washcloth bar.

Mounting: ​<< Recessed; or  Surface mounted>>​.

Material and Finish: ​​<< Stainless steel, No. 4 finish (satin); or  _______>>​​. ​​

Towel Bar:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Description: ​<< 3/4 inch square tube with rectangular end brackets;  3/4 inch round tube with circular end brackets; or  _______>>​.

Mounting: Flanges with ​<< concealed; or  exposed>>​ fasteners.

Length: ​​<< 18 inches;  24 inches;  30 inches; or  _______>>​​. ​​

Material and Finish: ​​<< Stainless steel, No. 4 finish (satin);  Stainless steel, No. 7 finish (polished);  Polished aluminum; or  _______>>​​. ​​

Towel Pin:

Manufacturer and Product: << Insert manufacturer’s name; product name or designation>>.

Description: Projecting minimum of << 3 inches; or  5 inches>> from wall surface.

Material and Finish: ​​<< Polished brass-plated zinc alloy (zamac);  Polished chrome-plated brass;  Stainless steel, No. 4 finish (satin);  Stainless steel, No. 7 finish (polished); or  _______>>​​. ​​

HIGH EFFICIENCY WARM AIR DRYERS

Manufacturers and Products:

American Dryer Inc.; << GXT Series; or  EXT Series>>.

Bradley Corporation; << 2902-280000;  2902-287300; or  2902-287400>>

Dyson Airblade; << Model AB02; or  Model AB04>>.

Excel Dryer Corporation; Xlerator

World Dryer Corporation; << SMARTdri; or  Airforce>>

Warm-Air Dryer:

Mounting: ​​<< Recessed;  Semi-recessed; or  Surface mounted>>​​​.

Operation: ​<< Touch button; or  Electronic-sensor>>​ activated with timed power cut-off switch.

Operation Time: << 30 to 40;  80; or  _______>>  seconds.

Cover Material and Finish: << _______>>.

Electrical Requirements: ​​<< 115 V, 13 A, 1500 W;  115 V, 15 A, 1725 W;  115 V, 20 A, 2300 W;  208-240 V, 9-10 A, 1900-2300 W; or  _______>>​​. ​​

UNDER LAVATORY GUARDS

Manufacturers:

Plumberex Specialty Products, Inc.

Truebro IPS Corporation.

​<< Insert manufacturer’s name>>​

Under Lavatory Guards:

Description: Insulating pipe covering for supply and drain piping assemblies that prevent direct contact with and burns from piping; allows service access without removing coverings.

Material and Finish: Antimicrobial, molded plastic, white.

CUSTODIAL ACCESSORIES

Manufacturers:

A & J Washroom Accessories, Inc.

American Specialties, Inc.

Bobrick Washroom Equipment, Inc.

Bradley Corporation.

GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

Tubular Specialties Manufacturing, Inc.

<< Insert manufacturer’s name>>.

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain paragraphs corresponding to custodial accessories selected for project.

      - ***************************************************************************************

Utility Shelf:

Description: With exposed edges turned down not less than 1/2 inch and supported by two triangular brackets welded to shelf underside.

Size: ​​<< 16 inches long by 6 inches deep; or  _______>>​​. ​​

Material and Finish: Not less than nominal 0.05 inch thick Stainless steel, No. 4 finish (satin).

Mop and Broom Holder:

Description: ​<< Unit with shelf, hooks, holders, and rod suspended beneath shelf; or  _______>>​.

Length: ​​<< 36 inches; or  _______>>​​. ​​

Hooks:​​<< Three; or  _______>>​​. ​​

Mop/Broom Holders: << Four; or  _______>>, spring-loaded, rubber hat, cam type.

Material and Finish: Stainless steel, No. 4 finish (satin).

Shelf: Not less than nominal 0.05 inch thick Stainless steel.

Rod: Approximately 1/4 inch diameter Stainless steel.

FABRICATION

Only an unobtrusive stamped logo of the manufacturer is permitted on exposed faces of the units. In an unexposed location, place the manufacturer’s name and product model number.

Fabricate units with welded and ground joints and edges rolled. Hang doors with continuous Stainless steel piano hinges. Provide for concealed anchorage wherever possible.

      - ***************************************************************************************

      - Retain the following paragraph when mirrors are in project-scope.

      - ***************************************************************************************

Mirror fabrication:

Fabricate frames to accommodate wood, felt, plastic, or other glass edge protection material.

Provide galvanized steel backing sheet, no less than 22 gage and full mirror size, with non-absorptive filler material. Corrugated cardboard is not an acceptable filler material.

Provide hanger system for tamper-proof installation, using galvanized steel wall brackets with concealed locking devices requiring a special tool to remove.

Keys: Provide universal keys for internal access to accessories for servicing and re-supplying. Provide minimum of << six; or  _______>> keys to LANL STR.

PART 3 - EXECUTION

    - **********************************************************************************************

      - Retain the following Article when air dryers are in project-scope.

    - **********************************************************************************************

PRE-INSTALLATION COORDINATION

Coordinate requirements for blocking to ensure adequate means for support and installation of hand dryers.

Coordinate requirements for power supply, conduit, disconnect switches, and wiring.

EXAMINATION

Do not begin installation until substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify LANL STR regarding unsatisfactory preparation before proceeding.

INSTALLATION

Install accessories in accordance with approved manufacturer’s installation instructions.

Securely attach each accessory, level and plumb, at locations shown on the Drawings and at heights, spacing, and load resistance required by <<ABA Standards>>.

Use fasteners that are appropriate for surfaces where accessories are to be mounted.

Use concealed fasteners wherever possible.

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Retain the first paragraph when mirrors are in project-scope. Retain the second paragraph when grab bars are in project-scope. Retain the third paragraph when dryers are in project-scope.

      - ***************************************************************************************

Secure mirrors in a tamper-proof manner.

Grab bars: Install to withstand a load in any direction of at least 250 lbf, when tested according to <<ASTM F446>>​.

Warm Air Dryers: Install electric warm air dryer in accordance with manufacturer’s written instructions, using fasteners appropriate to substrate indicated and recommended by the manufacturer. Install electric hand dryers level, plumb, and firmly anchored in locations and at heights indicated.

        - ******************************************************************************

        - Retain the following subparagraph when required for project.

        - ******************************************************************************

Install blocking per Section ​​<< 06 1000 - Rough Carpentry>>​​ in stud partitions for mounting hand dryers.

ADJUSTING AND CLEANING

Inspect installation to verify secure and proper mounting.

Adjust accessories for unencumbered, smooth operation. Replace damaged or defective items.

      - ***************************************************************************************

      - Retain the following paragraph when dryers are in project-scope.

      - ***************************************************************************************

Test each dryer to verify operation, control functions, and performance. Correct deficiencies.

Remove temporary labels and protective coatings.

Clean and polish exposed surfaces according to manufacturer’s written recommendations.

Clean stainless steel << and mirror ; or  None - N/A>>​surfaces in compliance with manufacturer’s instructions.

PROTECTION

Protect installed products until completion of project.

Touch-up, repair or replace damaged products before Substantial Completion.

SCHEDULES

      - ***************************************************************************************

      - Include schedule when selections of specific toilet accessories have been made.

      - Consider the following examples when developing Project schedule.

      - A.     Grab Bars 1: Satin stainless steel.

        - 1.      ABC Mfr: Model 1234.

        - 2.      DEF Mfr: Model 5678.

        - 3.      Provide at each location indicated in barrier-free toilet stalls.

      - ***************************************************************************************

​<< _______>>​

​<< _______>>​

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 10 2800, REV. 4, DATED APRIL 15, 2026.