Section 32 3100 
Fences and Gates

  - ************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 5 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Civil POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - ************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Include paragraphs based on project-scope. Typically installation of fences/gates require concrete work and electrical grounding.

      - ***************************************************************************************

Wire, stays, braces, posts, and accessories

Pedestrian and vehicle gates

Concrete

Electrical grounding

RELATED REQUIREMENTS

Section << 01 2500 - Substitution Procedures>>

Section << 03 3053 - Miscellaneous Cast-in-Place Concrete>>

Section << 31 2000 - Earth Moving>>

REFERENCE STANDARDS

ASTM A116 - Standard Specification for Metallic-Coated, Steel-Woven Wire Fence Fabric.

ASTM A121 - Standard Specification for Metallic-Coated Carbon Steel Barbed Wire.

ASTM A153/A153M - Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware.

ASTM A641/A641M - Standard Specification for Zinc-Coated (Galvanized) Carbon Steel Wire.

ASTM A702 - Standard Specification for Steel Fence Posts, Hot Wrought.

ASTM A1011/A1011M - Standard Specification for Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength.

ASTM B8 - Standard Specification for Concentric-Lay-Stranded Copper Conductors, Hard, Medium-Hard, or Soft.

ASTM F900 - Standard Specification for Industrial and Commercial Swing Gates.

FS RR-F-191/4 - Fencing, Wire and Post, Metal (Chain-Link Fence Accessories).

IEEE 80 - IEEE Guide for Safety in AC Substation Grounding.

IEEE C2 - National Electrical Safety Code(R) (NESC(R)).

LANL PERFORMED WORK

Lay out fence lines, gates, and terminal posts with suitable stakes (at intervals not exceeding 500 feet or line of sight).

Mark utility lines, USC&G benchmarks, property monuments, and other underground structures with suitable stakes.

Obtain excavation/soil disturbance permit for Subcontractor.

ACTION SUBMITTALS

      - ***************************************************************************************

      - Retain the following paragraph when fences/gates are in project-scope.

      - ***************************************************************************************

Catalog data on fabric, posts, wire, accessories, fittings, and hardware.

Two legible copies of batch tickets for each load of concrete to the LANL Subcontract Technical Representative (STR).

QUALITY ASSURANCE

Comply with the following unless otherwise noted.

        - ******************************************************************************

        - Retain the following subparagraphs based on project-scope. The first five subparagraphs apply to fences; the last subparagraph applies to gates.

        - ******************************************************************************

Accessories: <<FS RR-F-191/4>>

Stays: <<ASTM A116>>

Angles, braces: <<ASTM A1011/A1011M>>

Wire: <<ASTM A641/A641M>>

T-beam posts: <<ASTM A702>>

Gate Construction:  <<ASTM F900>>

DELIVERY, STORAGE, AND HANDLING

Deliver, store, and handle structural steel without damaging finish.

Deliver manufactured materials in original unopened packages, containers, or bundles with manufacturer's label intact and legible.

Store materials off ground, under cover, and away from damp surfaces.

Remove damaged, unlabeled or unsatisfactory materials that do not meet this Section from the job site.

PART 2 - PRODUCTS

    - **********************************************************************************************

      - The following Article Product Options and Substitutions is a designer-choice Retain when project allows for substitutions.

    - **********************************************************************************************

PRODUCT OPTIONS AND SUBSTITUTIONS

Alternate products may be accepted; follow Section ​​<< 01 2500 - Substitution Procedures>>​​.

MATERIALS

      - ***************************************************************************************

      - The following paragraphs are designer-choice. Include paragraphs based on project-scope. The first four paragraphs apply to fences; the last paragraph applies to gates. If allowed by ENV via ESM Civil POC, 12-1/2 gauge barbed wire can be substituted.

      - ***************************************************************************************

Fencing:

Angles, posts, braces and wire shall be steel.  Bolts, hardware and other parts shall be steel, malleable iron or ductile iron.

Fencing Materials:

Wire: << ASTM A641/A641M smooth; or  ASTM A121 barbed>> with a Class 1 zinc coating. Wire shall consist of << one; or  two>> strand(s) of No. << 9 single; or  12-1/2 double barbed>> gauge copper-bearing steel wire<< with large four point hard temper round barbs spaced approximately 5 inches apart (design 12-4-5-14R); or  None - N/A>>.

Tie wires for fastening wire to steel posts: No. 12 gauge copper-bearing steel wire.  Tie wires shall be heavily galvanized by the hot-dip process.

Stays: No. 9-1/2 gauge copper-bearing steel wire conforming to the requirements of <<ASTM A116>>.  Stays shall be 42 inches long.

Angles, Posts and Braces:

Angles and braces shall be fabricated from rail, billet or commercial grade steel per <<ASTM A1011/A1011M>>.  T-beam section posts shall be fabricated from rail, billet or commercial grade steel per <<ASTM A702>>.  Angles and braces shall be galvanized by the hot-dip process after fabrication.  Angles and braces per <<ASTM A1011/A1011M>> shall be triple coated with a minimum of 0.9 ounces per square foot of zinc, 15 micrograms per square inch of chromate and 0.3 mils of polyurethane finish.  T-beam section line posts per <<ASTM A702>> shall be painted with three (3) coats of an anti-corrosive aluminum paint or suitable substitute to prevent corrosion.

Corner and intermediate brace posts and braces shall be angles of the dimensions and length indicated on the Drawings.

Line posts shall have a minimum weight of 1.33 pounds per foot excluding the anchor plates.  Line posts shall be T-beam sections of the length indicated on the Drawings.  Line posts shall have corrugations, ribs or notches spaced at approximately one inch on center to hold the fence wire in the spacing designated on the Drawing.  Anchor plates shall have a minimum of 18 square inches of area and shall weigh not less than 2/3 pounds each.  They shall be welded or riveted to the section so as to prevent displacement when the posts are driven.

Fittings:

Fittings, hardware and appurtenances for fences shall be commercial quality steel, malleable iron or wrought iron galvanized per <<ASTM A153/A153M>>​.

Gates:

Gate fabric shall be No. 11 gauge copper bearing open-hearth steel wire, woven in a 2-inch mesh, and heavily galvanized by the hot-dip process after weaving.  Gates shall be 4-feet high and shall be furnished with pivot-type hinges and center stop. Gates shall provide clear openings as shown on Drawings.  Latches are required only if called out in the subcontract.

Gate Posts:

Gate Leaf - Widths up to 6 feet: 2-1/2 inch Schedule 40.

Gate Leaf - Widths up to 12 feet: 3-1/2 inch Schedule 40.

Gate Leaf - Widths up to 18 feet: 6 inch Schedule 40.

Gate Leaf - Widths up to 23 feet: 8 inch Schedule 40.

Gate Post Caps: Formed steel, malleable cast iron, or aluminum, sized to post diameter, with set screw retainer.

Gate Frames

Gate Leaf - Widths Less than 10 feet: 1-1/2 inch Schedule 40.

Gate Leaf - Widths 10 Feet to 16 feet: 2 inch Schedule 40.

Gate Leaf - Widths Greater than 16 feet: 2 inch Schedule 80.

FENCE GROUNDING

Grounding Cable: No 4/0 AWG bare, stranded, soft temper copper cable conforming to <<ASTM B8>>​.

Flexible Braid: Tinned copper braid with tinned copper ferrules; minimum 250 ampere rating; 12 inch minimum length. O-Z/Gedney Type FB.

Cable to Pipe Clamps: NRTL (National Recognized Testing Laboratory) listed copper alloy connectors with silicon bronze hardware for making cable to pipe connections. O-Z/Gedney Type ABG 1-1/2 inch and smaller, Type CG 2 inch and larger pipe diameter.

Flexible Braid to Pipe Clamps: NRTL listed copper alloy connectors with silicon bronze hardware for making braid or copper bar to pipe connections.  O-Z/Gedney Type RG.

CONCRETE

Furnish concrete for posts and braces in accordance with Section ​<< 03 3053 - Miscellaneous Cast-in-Place Concrete>>​.

PART 3 - EXECUTION

PREPARATION

Notify LANL Subcontract Technical Representative (STR) 10 working days prior to start of construction to identify known utilities and stake and flag locations.

      - ***************************************************************************************

      - Specify site specific clearing and grading requirements in the specification section or on the drawings. If specification section is selected, include those requirements in paragraph below.

      - ***************************************************************************************

Before installing fence, perform site clearing and grading << as noted on Drawings; or  per this Section>>​.

Allow footing to cure minimum 7 days before installing fabric and other materials.

GENERAL REQUIREMENTS

Once in place, peen or spot-weld gate hardware to prevent easy removal.

Coat damaged galvanized finish with zinc-enriched paint.

Leave area of installation neat and free of debris caused by erection of fence.

ELECTRICAL GROUNDING

      - ***************************************************************************************

      - Retain the first paragraph when gates are in project-scope. Retain the second and third paragraph when fences are in project-scope.

      - ***************************************************************************************

Bond gateposts on both sides of gate openings using direct buried grounding cable and cable to pipe clamps.  Bond gateposts to gates using flexible braid and flexible braid to pipe clamps.  Ground posts on both ends of gates; steel posts set in concrete will be considered as adequately grounded.

Ground permanent metallic fences crossed by overhead power at every third post for a distance of 50 feet from the crossing; chain link fences with steel post set in concrete will be considered as adequately grounded.

Ground metal fences surrounding substations and switching stations to station ground system in accordance with the <<IEEE C2>> (National Electrical Safety Code) and <<IEEE 80>>.

EXCAVATION, BACKFILL, AND COMPACTION

Refer to Section << 31 2000 - Earth Moving>>.

INSTALLATION

Concrete:

Concrete foundations for intermediate brace posts and corner posts shall be circular in horizontal section, not less than the angle dimension plus 9 inches in diameter and 3 feet 6 inches deep. Concrete foundations for braces shall be circular in horizontal section, not less than 24 inches in diameter and 12 inches deep.

Where the Drawings require that posts and braces be embedded in concrete, install temporary guys or braces such that posts and braces will be held in the proper position until the concrete has set sufficiently to hold them in the proper position.  No materials shall be installed on posts and braces set in concrete and no strain shall be placed on guys or bracing until seven days have elapsed from the time of placing the concrete.  Ensure that the concrete has sufficiently cured prior to pulling the wire.

      - ***************************************************************************************

      - Retain the following paragraph when fences/gates are in project-scope.

      - ***************************************************************************************

Posts:

All corner posts, line posts and intermediate brace posts shall be set plumb and shall be spaced in the line of the fence not to exceed 16-foot centers. All posts shall be set or driven to a depth not less than 36 inches.

Metal line posts may be driven.

Corner and intermediate brace posts shall be set in concrete.

Intermediate brace posts shall be placed at intervals not less than 100 feet and not more than 600 feet and shall be spaced evenly between corner and gate posts.

          - **********************************************************************

          - Retain the following subparagraph when gates are in project-scope.

          - **********************************************************************

Gate Posts: Set in concrete in holes of diameter and depth noted on Drawings in a vertical position, plumb within a one half inch tolerance over height of fence in two planes, and in line. Backfill concrete into excavation and extend 2 inches above grade. Crown concrete at top to shed water and slope away from post. Caps shall be installed on all pipe posts.

      - ***************************************************************************************

      - Retain the following two paragraphs when fences are in project-scope.

      - ***************************************************************************************

Wire:

Stretch fence wire by mechanical stretcher or other devices designed for this use.  Stretching by motor vehicle will not be permitted.  The length between pull posts shall not exceed 100 feet.

Stays:

Stays shall be twisted into place in accordance with the manufacturer's written instructions at the spacing indicated in the Drawings.

      - ***************************************************************************************

      - Retain the following paragraph when fences/gates are in project-scope.

      - ***************************************************************************************

Bolts and Hardware:

All nuts, bolts and tie wires shall be securely fastened to deter theft and vandalism and shall be placed to assure visual evidence of tampering.

Painting:

Surfaces that have been cut or filed or surfaces where the galvanized coating has been broken or damaged shall be painted with three coats of an anti-corrosive aluminum paint or suitable substitute to prevent corrosion.

Clearance:

Provide suitable closures approved by the Engineer of Record at irregularities in grade such as curbs or ditches. Vertical posts shall not exceed 6 inches open space to the adjacent post or solid structure.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the following reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 32 3100, REV. 5, DATED APRIL 15, 2026.