Section 32 3113 
Chain Link Fences and Gates

  - *************************************************************************************************************

    - LANL MASTER SPECIFICATION SECTION

Rev. 5 Summary of Changes: Edited and updated as required for import into SpecLink.

      - NOTE TO SPECIFIER ON USAGE IN SPECLINK: Activate the Section by turning on Seq #0000, followed by activating paragraphs in PART 1, Article SECTION INCLUDES. This will activate related articles, paras/subparas in the rest of the Section (i.e., SECTION INCLUDES in PART 1 activates articles and paras/subparas in PARTS 2 and 3). If there are choice fields to edit the entire Section (typically in this first author note or beginning of the Section), make selections at those locations, which will filter through the remaining Section. All Articles, paragraphs and subparagraphs must be activated sequentially from top to bottom, unless noted otherwise.

      - This template must be edited for each project. In doing so, Specifier must add job-specific requirements. Selections and choice fields are used to indicate designer choices or locations where text must be supplied by the designer. The section must also be edited to remove requirements for processes, items, or designs that are not included in the project.

      - To seek a variance from applicable requirements in this Section, contact the Engineering Standards Manual (ESM) Civil/Security POC  (LANL access only). Please contact POC with suggestions for improvement as well. LANL personnel should submit here: Suggested Change or Request Clarification; subcontractors via email to engstandards@lanl.gov.

      - When assembling a specification package, include applicable specification sections from all Divisions, especially Division 1, General Requirements.

      - Section is developed for ML-4 projects.  For ML-1, 2, and 3 applications, additional requirements and independent reviews should be added if increased confidence in procurement or execution is desired; see LANL ESM Chapter 1 - General, Section Z10 Specifications and Quality sections.

  - *************************************************************************************************************

PART 1 - GENERAL

SECTION INCLUDES

      - ***************************************************************************************

      - The following two paragraphs are designer-choice. Include paragraphs based on project-scope. Typically installation of fences/gates requires concrete work and electrical grounding.

      - ***************************************************************************************

Fence framework, fabric, and accessories

Pedestrian and vehicle gates

Concrete

Electrical grounding

RELATED REQUIREMENTS

Section << 03 3053 - Miscellaneous Cast-in-Place Concrete>>

Section << 31 2000 - Earth Moving>>

REFERENCE STANDARDS

ASTM A121 - Standard Specification for Metallic-Coated Carbon Steel Barbed Wire.

ASTM A123/A123M - Standard Specification for Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products.

ASTM A392 - Standard Specification for Zinc-Coated Steel Chain-Link Fence Fabric.

ASTM B8 - Standard Specification for Concentric-Lay-Stranded Copper Conductors, Hard, Medium-Hard, or Soft.

ASTM F552 - Standard Terminology Relating to Chain Link Fencing.

ASTM F567 - Standard Practice for Installation of Chain-Link Fence.

ASTM F626 - Standard Specification For Fence Fittings.

ASTM F900 - Standard Specification for Industrial and Commercial Swing Gates.

ASTM F1043 - Standard Specification for Strength and Protective Coatings on Steel Industrial Fence Framework.

ASTM F1083 - Standard Specification for Pipe, Steel, Hot-Dipped Zinc-Coated (Galvanized) Welded, for Fence Structures.

ASTM F1910 - Standard Specification for Long Barbed Tape Obstacles.

FS RR-F-191/1 - Fencing, Wire and Post Metal (Chain-Link Fence Fabric).

FS RR-F-191/2 - Fencing, Wire and Post, Metal (Chain-Link Fence Gates) (Detail Specification).

FS RR-F-191/3 - Fencing, Wire and Post, Metal (Chain-Link Fence Posts, Top Rails and Braces) (Detail Specification).

FS RR-F-191/4 - Fencing, Wire and Post, Metal (Chain-Link Fence Accessories).

IEEE 80 - IEEE Guide for Safety in AC Substation Grounding.

IEEE C2 - National Electrical Safety Code(R) (NESC(R)).

LANL PERFORMED WORK

Layout fence lines, gates, and terminal posts with suitable stakes (at intervals not exceeding 500 feet or line of sight).

Layout USC&G benchmarks, property monuments, and other underground structures with suitable stakes.

Obtain excavation/soil disturbance permit for Subcontractor.

Furnish signs for Subcontractor to install on fence.  See Article << 3.12; or  _______>> Sign Installation (GFE) in PART 3.

ACTION SUBMITTALS

      - ***************************************************************************************

      - Retain the following paragraph when fences/gates are in project-scope.

      - ***************************************************************************************

Catalog data on fabric, posts, accessories, fittings, and hardware.

Two legible copies of batch tickets for each load of concrete to the LANL Subcontract Technical Representative (STR).

QUALITY ASSURANCE

Comply with the following unless otherwise noted.

        - ******************************************************************************

        - Retain the following subparagraphs based on project-scope. The first subparagraph is a designer-choice. The second through ninth subparagraphs apply to fences; the last two subparagraphs apply to gates.

        - ******************************************************************************

Definition of Terms: <<ASTM F552>>

Fabric: <<FS RR-F-191/1>>

Posts, Rails and Braces: <<FS RR-F-191/3>>

Accessories: <<FS RR-F-191/4>>

Fence Fittings: <<ASTM F626>>

Steel Pipe for Fence: <<ASTM F1083>>

        - ******************************************************************************

        - Selecting barbed tape here will select all related articles/paras/subparas in Section. Work with LANL Physical Security to determine whether barbed wire and/or barbed tape are needed.

        - ******************************************************************************

Long Barbed Tape: <<ASTM F1910>>

Installation: <<ASTM F567>>

Strength Requirements: <<ASTM F1043>>

Gates: <<FS RR-F-191/2>>

Gate Construction: <<ASTM F900>>

DELIVERY, STORAGE, AND HANDLING

Deliver, store, and handle structural steel without damaging finish.

Deliver manufactured materials in original unopened packages, containers, or bundles with manufacturer's label intact and legible.

Store materials off ground, under cover, and away from damp surfaces.

Remove damaged unlabeled or unsatisfactory materials that do not meet this Section from the job site.

PART 2 - PRODUCTS

MATERIALS

      - ***************************************************************************************

      - Some secured areas may require 9 AWG (0.148) inch fabric. Per NNSA NAP 70.2 for protection of a LA or higher (PA) chain link fabric requires a No. 11 AWG or heavier galvanized steel wire or equivalent protection as defined in the SSP. Consult with LANL Physical Security. Retain the following paragraph when fences/gates are in project-scope.

      - ***************************************************************************************

Fabric: <<ASTM A392>>, Class 1, zinc coated (1.2 ounces), steel wire/fabric, 2 inch mesh size on a side, << 11 American Wire Gauge (AWG) (0.120 inches); or  9 American Wire Gauge (AWG) (0.148 inches)>> coated wire size, galvanized before weaving, with twisted barbed selvages top and bottom.

      - ***************************************************************************************

      - Retain the following paragraph when fences are in project-scope.

      - ***************************************************************************************

Posts, Rails & Braces: <<ASTM F1083>> galvanized steel pipe.

Galvanizing: <<ASTM A123/A123M>>​, unless otherwise indicated, provide hot-dipped, zinc-coated accessories of ferrous material with weight of zinc coating not less than 1.2 ounces per square foot.

      - ***************************************************************************************

      - The installation o​​​​f outriggers with barbed wire or barbed tape only applies to a Protected Area or other defined higher security area. The installation of a fence top consisting of the outriggers and wire is not required on a typical Limited Area fence. Reference VAR-2011-089 dated 1/26/11. Consult with LANL Physical Security.

      - Selecting barbed wire here will select all related articles/paras/subparas in Section. Work with LANL Physical Security to determine whether barbed wire and/or barbed tape are needed.

      - ***************************************************************************************

Barbed Wire: <<ASTM A121>> metallic coated steel, Type Z zinc coating strand wire, 12-1/2 AWG (0.099 inches) steel wire, 4 barb points, 14 AWG (0.080 inches) spaced on 5 inch centers.

      - ***************************************************************************************

      - Retain the following paragraph when barbed tape is in project-scope.

      - ***************************************************************************************

Barbed Tape: 430 stainless steel, 0.025 inches thick by minimum 1-inch wide prior to forming, 18 inch diameter single helical coil, die stamped to produce a cluster of 4 barbed points at maximum 4 inches on center, cold rolled around a galvanized steel core wire. Aluminum ties not acceptable.

      - ***************************************************************************************

      - Retain the following paragraph when fences are in project-scope.

      - ***************************************************************************************

Fence Fittings: <<ASTM F626>>, type and components as noted.

      - ***************************************************************************************

      - Retain the following Article when fences/gates are in project-scope.

      - ***************************************************************************************

BASIC FENCE SYSTEM COMPONENTS

Terminal Posts (angles, corners, ends, and pull posts): 2-1/2 inch Schedule 40 for fence height up to 8 feet and 3 inch Schedule 40 for fence height over 8 feet and equal to or under 16 feet.

Line Posts: 2 inch Schedule 40 for fabric height up to 8 feet, 2-1/2 inch Schedule 40 for fabric height over 8 feet and equal to or under 16 feet.

Braces at Terminal or Gateposts: 1-1/4 inch Schedule 40.

Tension Rods: 3/8 inch diameter galvanized steel with turnbuckle end-fitting-type tighteners.

Tension Wire: 7- AWG coil spring, hard tempered carbon steel wire.

Tension Bars: 3/4 x 3/16 inch thick galvanized steel. Minimum length shall be 2 inches less than the full fabric height.

Tension Bands: Minimum << 3/4 x 0.074 inch; or  _______>> galvanized steel offset bands.

Brace Bands: Minimum << 3/4 x 0.105 inch; or  _______>> thick galvanized steel.

Brace Ends: Cupped fittings of formed steel or cast iron with ears for attaching horizontal braces to brace bands and for connecting diagonal tension rods.

Wire Ties: Galvanized, steel 9 AWG with a diameter of 0.148 inches.

Post Caps: Formed steel, malleable cast iron, or aluminum, sized to post diameter, with set screw retainer.

Hog Rings: Minimum 12 AWG galvanized steel

      - ***************************************************************************************

      - Retain the following paragraph when barbed wire is in project-scope.

      - ***************************************************************************************

Barbed Wire Extension Arms: Galvanized pressed steel, type as specified.

      - ***************************************************************************************

      - Retain the following three paragraphs when gates are in project-scope.

      - ***************************************************************************************

Gate Posts:

Gate leaf-widths up to 6 feet: 2-1/2 inch Schedule 40.

Gate leaf-widths up to 12 feet: 3-1/2 inch Schedule 40.

Gate leaf-widths up to 18 feet: 6 inch Schedule 40.

Gate leaf-widths up to 23 feet: 8 inch Schedule 40.

Gate Frames:

Gate leaf-widths less than 10 feet: 1-1/2 inch Schedule 40.

Gate leaf-widths 10 feet to 16 feet: 2 inch Schedule 40.

Gate leaf-widths greater than 16 feet: 2 inch Schedule 80.

Diagonal tension rods for leaf-widths over 6 feet.

For gate leaves greater than 8 feet in any direction, provide intermediate braces placed symmetrically so that frame members, including bracing, are spaced not further than 8 feet. Provide braces same size as those called out for fence.

Provide non-lift-off type gate hinges sized for gate of adequate strength with large rearing surface for clamping in position so that hinges do not easily twist or turn with gate action.

FENCE GROUNDING

Grounding Cable:  No. 4/0 AWG bare, stranded, soft temper copper cable conforming to <<ASTM B8>>.

Flexible Braid:  Tinned copper braid with tinned copper ferrules; minimum 250 ampere rating; 12-inch-minimum length.  O-Z/Gedney Type FB.

Cable to Pipe Clamps:  NRTL (National Recognized Testing Laboratory) listed copper alloy connectors with silicon bronze hardware for making cable to pipe connections. O-Z/Gedney Type ABG 1-1/2 inch and smaller, Type CG 2 inch and larger pipe diameter.

Flexible Braid to Pipe Clamps:  NRTL listed copper alloy connectors with silicon bronze hardware for making braid or copper bar to pipe connections.  O-Z/Gedney Type RG.

CONCRETE

Concrete: Furnish concrete for posts and braces in accordance with Section << 03 3053 - Miscellaneous Cast-in-Place Concrete>>​.

PART 3 - EXECUTION

PREPARATION

Notify LANL STR 15 working days prior to start of construction to identify known utilities and stake and flag locations.

      - ***************************************************************************************

      - Specify site specific clearing and grading requirements in the specification section or on the drawings. If specification section is selected, include those requirements in paragraph below.

      - ***************************************************************************************

Before installing chain-link fence, perform site clearing and grading << as noted on Drawings; or  per this Section>>.

Allow footing to cure minimum 7 days before installing fabric and other materials.

GENERAL REQUIREMENTS

Coat damaged galvanized finish with zinc-enriched paint.

Leave area of installation neat and free of debris caused by erection of fence.

      - ***************************************************************************************

      - Retain the following paragraph when fences are in project-scope.

      - ***************************************************************************************

Posts, bracing, and other structural members shall be located on the inside of the security fence.

      - ***************************************************************************************

      - Retain the following paragraph when gates are in project-scope.

      - ***************************************************************************************

Gate hardware installed in PA or higher security area that if removed would facilitate unauthorized entry must be installed in a manner to mitigate tampering and/or removal (e.g., by brazing, peening, or welding). Consult with LANL Physical Security.

      - ***************************************************************************************

      - Retain the following Article when gates are in project-scope.

      - ***************************************************************************************

LATCHES

Single gates less than 10 feet wide may use forked type latches.

Provide single gates, 10 feet, or more, wide, and inactive leaves of double gates with positive latching devices at top, bottom and center of closing edges such as fork type latches with full gate height plunger bar or rod.

At double gates, provide locking devices that retain both gate leaves in same plane when closed.

Arrange plunger bars and rods so that they engage gate stops and cannot be raised when locked.

Rigidly weld brackets for plunger bars holders to inactivate leaves.

Arrange latching mechanisms at double gates so one padlock can lock both gate leaves at center latch integral to gate.

Install keepers consisting of a mechanical device for securing free end of gate when in full open position.

      - ***************************************************************************************

      - Retain the following Article when fences/gates are in project-scope.

      - ***************************************************************************************

POSTS

Space line posts equidistant at intervals not exceeding 10 feet. Measure interval parallel to grade of proposed fence and in line of fence from center to center of post.

Set terminal posts (end, corner, and gate) at beginning and end of each continuous length of fence and at abrupt changes in vertical and horizontal alignments.

      - ***************************************************************************************

      - The following post footing dimensions are based on normal soil conditions.  Verify actual soil conditions with LANL Structural Engineer.

      - ***************************************************************************************

Set fence and gate posts in concrete in holes of diameter and depth as follows:

Minimum Diameter: Four times outside diameter of post up to four inches and three times for larger outside diameters.

Minimum Depth: 36 inches plus an additional 3 inches for each 1 foot increase in fence height over 4 feet.

Post shall be centered in the concrete base within a +/- one inch tolerance.

Set fence posts in a vertical position, plumb within a one half inch tolerance over height of fence in two planes, and in line.  Backfill concrete into excavation and extend 2 inches above grade.  An alternative method is to stop footing 4 inches below grade to allow for cover with sod, black top, or concrete slab.  In either case, crown concrete at top to shed water and slope  2 inches away from post to ground with tolerance of +0 to -1 inch. Crown shall have a troweled finish and be free from surface contours that trap water.

When solid rock or concrete is encountered, without an overburden of soil, set posts in solid rock or concrete.  Depth of hole shall be twelve times the largest cross section of posts. Diameter of hole shall be 6 inch greater than largest cross section of post.

The use of sleeves in order to leave voids in new concrete construction is recommended.

Half-fill the void with non-shrinkable hydraulic cement and force post to bottom of hole and plumb.  Thoroughly work additional grout into hole so as to leave no voids.  Crown grout to shed water.

Provide tension offset bands fitted around terminal posts at maximum 15 inch intervals to attach tension bars to posts.

Provide brace center band to secure brace ends and tension rods to post.

Install post caps on all pipe post terminals: end, corner, and gate and all line posts that do not have barbed-wire extension arms.

      - ***************************************************************************************

      - Retain the following Article when fences/gates are in project-scope.

      - ***************************************************************************************

FABRIC

Place chain-link fabric on outside of the area enclosed.  Posts, bracing, and other structural members shall be located on the inside of the fence.

Place fabric by securing one end, applying sufficient tension to remove slack before making attachment elsewhere.  Tighten fabric to provide smooth uniform appearance free from sag.

Cut fabric by untwisting a picket and attach each span independently at terminal posts. Use stretcher bars with tension bands at maximum 15-inch intervals.

Join rolls of wire fabric by weaving a single picket into ends of rolls to form continuous mesh.

Provide continuous length tension bars equal to fence height and located wherever chain link fabric end attaches to terminal post.  Thread bars through fabric ends for full height, and attach to posts by tension bands.

Install fence fabric 2 inches maximum above ground level.  Ground level shall be defined as compacted ground or earth material or base course below the fence fabric.  Compacted material shall extend three feet on each side of fence and along same contour as fence within +/-6 inches.

Fasten fence fabric with wire ties to top and bottom of line posts and at intervals not exceeding 15 inches.  The final process of tightening the tie wire shall draw the fabric tightly to the post and top rail.  Wire ties shall wrap a full 360 degrees around the post and engage a full diamond of the fence fabric.  The two ends of the tie shall be twisted securely with three full twists, power twisting is subcontractor option.  Ends shall be cut off to prevent untwisting by hand.  Care shall be taken to assure ends of ties do not protrude beyond the vertical plane of the fabric.

      - ***************************************************************************************

      - Retain the following Article when barbed wire is required on the project. Work with LANL Physical Security.

      - ***************************************************************************************

BARBED WIRE

      - ***************************************************************************************

      - LANL Physical Security will determine if barbed wire (# of strands and type), outriggers, and/or barbed tape coil are required. For PA or higher security area, fencing must be topped by three or more strands of barbed wire single or double outriggers. Double outriggers may be topped with coiled barbed wire. Note: The aforementioned requirements do NOT apply to Limited Area or lower security areas.

      - ***************************************************************************************

Fencing: Top fencing with ​<< 3; or  _______>>​ strands of barbed wire on each extension arm.  Provide ​<< vertical extension arm outriggers;  single 45 degree extension arm; or  double 45 degree extension arm>>​.  Angle single extension arm away from security area or as directed by LANL Physical Security.

Pull taut to remove sag, firmly install barbed wire in slots of extension arms, and secure to post or terminal arm.

      - ***************************************************************************************

      - Retain the following paragraph when gates are in project-scope.

      - ***************************************************************************************

Gate: Install barbed wire strands at 6 inches on center between extended gate frame members above gate fabric.

      - ***************************************************************************************

      - Retain the following Article when barbed tape is required on the project. Work with LANL Physical Security.

      - ***************************************************************************************

BARBED TAPE

      - ***************************************************************************************

      - LANL Physical Security will determine if barbed tape is required and will adjust loop spacing.

      - ***************************************************************************************

Top fencing with barbed tape at loop spacing of 9 inches.

Fencing: Provide 45 degree double extension arm with 2 strands of barbed wire in each extension arm.  Pull wire taut to remove sag, and firmly install barbed wire in most inner and outer slot of extension arm.  Attach barbed tape to barbed wire with hog rings.

      - ***************************************************************************************

      - Retain the following paragraph when gates are in project-scope.

      - ***************************************************************************************

Gate: Provide one strand of barbed wire to top of extended gate frame and pull taut to remove sag.  Secure barbed tape to barbed wire and top selvage of gate fabric with hog rings at each loop. Barbed tape loop spacing same as fence.

      - ***************************************************************************************

      - Retain the following Article when gates are in project-scope.

      - ***************************************************************************************

GATES

Install gates true to opening and plumb in closed position within a vertical tolerance of one half inch.

Hang gates so that bottom of gate is as close to ground as practical (2 inches max.) while allowing sufficient clearance for free operation through at least 90 degree in one direction from closed position.

Fasten gate fabric to vertical (end) gate frame members using tension bars and bands as for fence fabric.  Fasten fabric to top and bottom gate frame members and to intermediate braces with 11 AWG wire ties or clips at minimum spacing of 14 inches on center.

      - ***************************************************************************************

      - Retain the following paragraph when barbed wire is in project-scope.

      - ***************************************************************************************

Extend end frame members 18 inches vertically above top member of gate frame to support barbed wire.

Provide tension rods as diagonal braces on gates and secure rods at gate corner only.

Gates installed in PA or higher security area, gate hardware shall be installed in a manner to mitigate tampering and/or removal, e.g., by brazing, peening or welding to prevent removal with hand tools.  Hinges shall be tamper proof and installed to prevent the gate from being lifted off.

      - ***************************************************************************************

      - Retain the following Article when fences are in project-scope.

      - ***************************************************************************************

TENSION WIRE AND RAIL

      - ***************************************************************************************

      - LANL Physical Security will determine if top tension wire or top rail is required. Select "top and" option in the two paragraphs below if top tension wire is required for the project.

      - ***************************************************************************************

Provide<< top and; or  None - N/A>> bottom tension wire and stretch wire from end to end of each stretch of fence at height that will enable it to be fastened to the fabric.

Stretch tension wire taut (not to exceed 6 inches sideway deflection) between terminal posts, securely fasten the tension wire to the terminal posts. Tension wire shall be within 2 inches of<< top and; or  None - N/A>> bottom of the fence fabric and secured to fabric with hog rings at 24 inches on center, secure with wire ties to every third line post minimum.

      - ***************************************************************************************

      - Retain the paragraph below if top rail is required for the project.

      - ***************************************************************************************

Provide top rail and support at each post so that a continuous brace from end to end of each stretch of fence is formed. Fasten fabric to the top rail at intervals not exceeding 24 inches. Securely fasten top rail to terminal posts and join with sleeves or coupling to allow for expansion and contraction. The rail sleeve must be fabricated to prevent movement along the rail.

ELECTRICAL GROUNDING

      - ***************************************************************************************

      - Retain the first paragraph when gates are in project-scope. Retain the second and third paragraph when fences are in project-scope.

      - ***************************************************************************************

Bond gateposts on both sides of gate openings using direct buried grounding cable and cable to pipe clamps.  Bond gateposts to gates using flexible braid and flexible braid to pipe clamps.  Ground posts on both ends of gates; steel posts set in concrete will be considered as adequately grounded.

Ground permanent metallic fences crossed by overhead power at every third post for a distance of 50 feet from the crossing; chain link fences with steel post set in concrete will be considered as adequately grounded.

Ground metal fences surrounding substations and switching stations to station ground system in accordance with the <<IEEE C2>> (National Electrical Safety Code) and <<IEEE 80>>.

EXCAVATION, BACKFILL, AND COMPACTION

Refer to Section << 31 2000 - Earth Moving>>.

SIGN INSTALLATION (GFE)

Install government furnished property signs or no trespassing signs in areas as indicated by the LANL STR.

Signs will generally be 24 inches x 24 inches or smaller and shall be attached to the wire fabric by the use of, as specified, hog rings.

General sign locations will be located on all gates, 25 feet each side of gates, trail crossings, and at visible distances along the fence line, not to exceed 300 feet.

END OF SECTION

    - **********************************************************************************************

      - Do not omit the fol​​​​lowing reference information:

    - **********************************************************************************************

THE FOLLOWING STATEMENT IS FOR LANL USE ONLY

THIS PROJECT SPECIFICATION SECTION IS BASED ON LANL MASTER SPECIFICATION SECTION 32 3113, REV. 5, DATED APRIL 15, 2026.